➜  ~ ssh -o ConnectTimeout=3 root@192.168.137.226

root@192.168.137.226's password:

Welcome to Ubuntu 20.04.6 LTS (GNU/Linux 5.10.110 aarch64)

* Documentation:  https://help.ubuntu.com

* Management:     https://landscape.canonical.com

* Support:        https://ubuntu.com/pro

Last login: Wed Nov 22 05:10:35 2023

-bash: /opt/ros/noetic/setup.bash: No such file or directory

-bash: /opt/ros/noetic/setup.bash: No such file or directory

root@talowe-rk3588:~#   # 1. 系统信息

root@talowe-rk3588:~#   uname -a

Linux talowe-rk3588 5.10.110 #49 SMP Thu Mar 14 19:05:30 CST 2024 aarch64 aarch64 aarch64 GNU/Linux

root@talowe-rk3588:~#   cat /etc/os-release

NAME="Ubuntu"

VERSION="20.04.6 LTS (Focal Fossa)"

ID=ubuntu

ID_LIKE=debian

PRETTY_NAME="Ubuntu 20.04.6 LTS"

VERSION_ID="20.04"

HOME_URL="https://www.ubuntu.com/"

SUPPORT_URL="https://help.ubuntu.com/"

BUG_REPORT_URL="https://bugs.launchpad.net/ubuntu/"

PRIVACY_POLICY_URL="https://www.ubuntu.com/legal/terms-and-policies/privacy-policy"

VERSION_CODENAME=focal

UBUNTU_CODENAME=focal

root@talowe-rk3588:~#   # 1. 系统信息

root@talowe-rk3588:~#   uname -a

Linux talowe-rk3588 5.10.110 #49 SMP Thu Mar 14 19:05:30 CST 2024 aarch64 aarch64 aarch64 GNU/Linux

root@talowe-rk3588:~#   cat /etc/os-release

NAME="Ubuntu"

VERSION="20.04.6 LTS (Focal Fossa)"

ID=ubuntu

ID_LIKE=debian

PRETTY_NAME="Ubuntu 20.04.6 LTS"

VERSION_ID="20.04"

HOME_URL="https://www.ubuntu.com/"

SUPPORT_URL="https://help.ubuntu.com/"

BUG_REPORT_URL="https://bugs.launchpad.net/ubuntu/"

PRIVACY_POLICY_URL="https://www.ubuntu.com/legal/terms-and-policies/privacy-policy"

VERSION_CODENAME=focal

UBUNTU_CODENAME=focal

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 2. NPU 驱动

root@talowe-rk3588:~#   ls -la /dev/rknpu*

ls: cannot access '/dev/rknpu*': No such file or directory

root@talowe-rk3588:~#   cat /sys/kernel/debug/rknpu/version 2>/dev/null || echo "No debug info"

RKNPU driver: v0.8.2

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 3. 网卡状态

root@talowe-rk3588:~#   ip link show

1: lo: <LOOPBACK,UP,LOWER_UP> mtu 65536 qdisc noqueue state UNKNOWN mode DEFAULT group default qlen 1000

link/loopback 00:00:00:00:00:00 brd 00:00:00:00:00:00

2: can0: <NOARP,ECHO> mtu 16 qdisc noop state DOWN mode DEFAULT group default qlen 10

link/can

3: can1: <NOARP,ECHO> mtu 16 qdisc noop state DOWN mode DEFAULT group default qlen 10

link/can

4: eth0: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc mq state DOWN mode DEFAULT group default qlen 1000

link/ether b6:4c:e9:3b:97:32 brd ff:ff:ff:ff:ff:ff

5: eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc mq state UP mode DEFAULT group default qlen 1000

link/ether ba:4c:e9:3b:97:32 brd ff:ff:ff:ff:ff:ff

6: docker0: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc noqueue state DOWN mode DEFAULT group default

link/ether 02:42:e6:e5:1a:7f brd ff:ff:ff:ff:ff:ff

root@talowe-rk3588:~#   ip addr show

1: lo: <LOOPBACK,UP,LOWER_UP> mtu 65536 qdisc noqueue state UNKNOWN group default qlen 1000

link/loopback 00:00:00:00:00:00 brd 00:00:00:00:00:00

inet 127.0.0.1/8 scope host lo

valid_lft forever preferred_lft forever

inet6 ::1/128 scope host

valid_lft forever preferred_lft forever

2: can0: <NOARP,ECHO> mtu 16 qdisc noop state DOWN group default qlen 10

link/can

3: can1: <NOARP,ECHO> mtu 16 qdisc noop state DOWN group default qlen 10

link/can

4: eth0: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc mq state DOWN group default qlen 1000

link/ether b6:4c:e9:3b:97:32 brd ff:ff:ff:ff:ff:ff

5: eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc mq state UP group default qlen 1000

link/ether ba:4c:e9:3b:97:32 brd ff:ff:ff:ff:ff:ff

inet 192.168.137.226/24 brd 192.168.137.255 scope global dynamic noprefixroute eth1

valid_lft 604282sec preferred_lft 604282sec

inet6 fe80::8bc1:203c:1ba1:fe37/64 scope link noprefixroute

valid_lft forever preferred_lft forever

6: docker0: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc noqueue state DOWN group default

link/ether 02:42:e6:e5:1a:7f brd ff:ff:ff:ff:ff:ff

inet 172.17.0.1/16 brd 172.17.255.255 scope global docker0

valid_lft forever preferred_lft forever

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 4. 内存

root@talowe-rk3588:~#   free -h

total        used        free      shared  buff/cache   available

Mem:          7.7Gi       170Mi       7.1Gi        18Mi       378Mi       7.4Gi

Swap:            0B          0B          0B

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 5. Python 环境

root@talowe-rk3588:~#   python3 --version

Python 3.8.10

root@talowe-rk3588:~#   pip3 --version

pip 20.0.2 from /usr/lib/python3/dist-packages/pip (python 3.8)

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查 NPU 模块

root@talowe-rk3588:~#   lsmod | grep rknpu

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 尝试加载模块

root@talowe-rk3588:~#   sudo modprobe rknpu

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查设备节点

root@talowe-rk3588:~#   ls -la /dev/rknpu*

ls: cannot access '/dev/rknpu*': No such file or directory

root@talowe-rk3588:~#   # 检查 dmesg 日志

root@talowe-rk3588:~#   dmesg | grep -i npu

pu

# 检查是否有 rknn 服务

systemctl status rknn* 2>/dev/null || echo "No rknn service"[    3.314855] rk_gmac-dwmac fe1c0000.ethernet: clock input or output? (output).

[    3.448812] rk_gmac-dwmac fe1b0000.ethernet: clock input or output? (output).

[    3.748876] input: rk805 pwrkey as /devices/platform/feb20000.spi/spi_master/spi2/spi2.0/rk805-pwrkey.3.auto/input/input0

[    3.770376] vdd_npu_s0: supplied by vcc4v0_sys

[    3.774447] vdd_npu_s0: 550 <--> 950 mV at 800 mV, enabled

[    4.760951] rkisp_hw fdcb0000.rkisp: max input:0x0@0fps

[    4.863761] input: rockchip-hdmi0 rockchip-hdmi0 as /devices/platform/hdmi0-sound/sound/card0/input1

[    4.865962] input: headset-keys as /devices/platform/nau8822-sound/input/input2

[    4.973249] input: rockchip-nau8822 Headset as /devices/platform/nau8822-sound/sound/card1/input3

[    5.059844] input: adc-keys as /devices/platform/adc-keys/input/input4

[    5.064893] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up npu-supply from device tree

[    5.064913] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up npu-supply property in node /power-management@fd8d8000/power-controller failed

[    5.071027] RKNPU fdab0000.npu: Adding to iommu group 0

[    5.071162] RKNPU fdab0000.npu: RKNPU: rknpu iommu is enabled, using iommu mode

[    5.071271] RKNPU fdab0000.npu: Looking up rknpu-supply from device tree

[    5.071954] RKNPU fdab0000.npu: Looking up mem-supply from device tree

[    5.072519] RKNPU fdab0000.npu: can't request region for resource [mem 0xfdab0000-0xfdabffff]

[    5.072541] RKNPU fdab0000.npu: can't request region for resource [mem 0xfdac0000-0xfdacffff]

[    5.072558] RKNPU fdab0000.npu: can't request region for resource [mem 0xfdad0000-0xfdadffff]

[    5.073034] [drm] Initialized rknpu 0.8.2 20220829 for fdab0000.npu on minor 1

[    5.073337] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up nputop-supply from device tree

[    5.073360] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up nputop-supply property in node /power-management@fd8d8000/power-controller failed

[    5.073435] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up npu1-supply from device tree

[    5.073454] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up npu1-supply property in node /power-management@fd8d8000/power-controller failed

[    5.073531] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up npu2-supply from device tree

[    5.073550] rockchip-pm-domain fd8d8000.power-management:power-controller: Looking up npu2-supply property in node /power-management@fd8d8000/power-controller failed

[    5.073680] RKNPU fdab0000.npu: Looking up rknpu-supply from device tree

[    5.073716] vdd_npu_s0: could not add device link fdab0000.npu: -EEXIST

[    5.073720] vdd_npu_s0: Failed to create debugfs directory

[    5.074250] RKNPU fdab0000.npu: Looking up mem-supply from device tree

[    5.074266] vdd_npu_s0: could not add device link fdab0000.npu: -EEXIST

[    5.074269] vdd_npu_s0: Failed to create debugfs directory

[    5.074897] RKNPU fdab0000.npu: Looking up rknpu-supply from device tree

[    5.074913] vdd_npu_s0: could not add device link fdab0000.npu: -EEXIST

[    5.074916] vdd_npu_s0: Failed to create debugfs directory

[    5.076698] RKNPU fdab0000.npu: bin=2

[    5.076884] RKNPU fdab0000.npu: leakage=9

[    5.076953] RKNPU fdab0000.npu: Looking up rknpu-supply from device tree

[    5.076972] debugfs: Directory 'fdab0000.npu-rknpu' with parent 'vdd_npu_s0' already present!

[    5.084176] RKNPU fdab0000.npu: pvtm=911

[    5.088268] RKNPU fdab0000.npu: pvtm-volt-sel=5

[    5.089175] RKNPU fdab0000.npu: avs=0

[    5.089372] RKNPU fdab0000.npu: l=10000 h=85000 hyst=5000 l_limit=0 h_limit=800000000 h_table=0

[    5.099697] RKNPU fdab0000.npu: failed to find power_model node

[    5.099713] RKNPU fdab0000.npu: RKNPU: failed to initialize power model

[    5.099721] RKNPU fdab0000.npu: RKNPU: failed to get dynamic-coefficient

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查内核模块是否存在

root@talowe-rk3588:~#   find /lib/modules/$(uname -r) -name "*rknpu*"

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查 rknpu 设备树

root@talowe-rk3588:~#   ls -la /sys/devices/platform/ | grep npu

drwxr-xr-x   5 root root    0 Nov 22  2023 fdab0000.npu

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查是否有 rknn 服务

root@talowe-rk3588:~#   systemctl status rknn* 2>/dev/null || echo "No rknn service"

root@talowe-rk3588:~#   # 检查 DRM 设备（NPU 用 DRM 子系统）

root@talowe-rk3588:~#   ls -la /dev/dri/

/device/uevent 2>/dev/null

# 检查 NPU 平台设备

ls -la /sys/devices/platform/fdab0000.npu/

# 检查 rknnlite 是否已安装

python3 -c "from rknnlite.api import RKNNLite; print('RKNNLite OK')" 2>&1total 0

drwxr-xr-x   3 root root        140 Nov 22  2023 .

drwxr-xr-x  15 root root       5440 Nov 22  2023 ..

drwxr-xr-x   2 root root        120 Nov 22  2023 by-path

crw-rw----+  1 root video  226,   0 Nov 22  2023 card0

crw-rw----+  1 root video  226,   1 Nov 22  2023 card1

crw-rw----+  1 root render 226, 128 Nov 22  2023 renderD128

crw-rw----+  1 root render 226, 129 Nov 22  2023 renderD129

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查 renderD 设备

root@talowe-rk3588:~#   cat /sys/class/drm/renderD*/device/uevent 2>/dev/null

DRIVER=rockchip-drm

OF_NAME=display-subsystem

OF_FULLNAME=/display-subsystem

OF_COMPATIBLE_0=rockchip,display-subsystem

OF_COMPATIBLE_N=1

MODALIAS=of:Ndisplay-subsystemT(null)Crockchip,display-subsystem

DRIVER=RKNPU

OF_NAME=npu

OF_FULLNAME=/npu@fdab0000

OF_COMPATIBLE_0=rockchip,rk3588-rknpu

OF_COMPATIBLE_N=1

MODALIAS=of:NnpuT(null)Crockchip,rk3588-rknpu

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查 NPU 平台设备

root@talowe-rk3588:~#   ls -la /sys/devices/platform/fdab0000.npu/

total 0

drwxr-xr-x   5 root root    0 Nov 22  2023 .

drwxr-xr-x 270 root root    0 Nov 22  2023 ..

drwxr-xr-x   3 root root    0 Nov 22  2023 devfreq

lrwxrwxrwx   1 root root    0 Nov 22  2023 driver -> ../../../bus/platform/drivers/RKNPU

-rw-r--r--   1 root root 4096 Jan  7 14:16 driver_override

drwxr-xr-x   4 root root    0 Nov 22  2023 drm

lrwxrwxrwx   1 root root    0 Jan  7 14:16 iommu -> ../fdab9000.iommu/iommu/fdab9000.iommu

lrwxrwxrwx   1 root root    0 Jan  7 14:16 iommu_group -> ../../../kernel/iommu_groups/0

-r--r--r--   1 root root 4096 Jan  7 14:16 modalias

lrwxrwxrwx   1 root root    0 Jan  7 14:16 of_node -> ../../../firmware/devicetree/base/npu@fdab0000

drwxr-xr-x   2 root root    0 Jan  7 14:16 power

lrwxrwxrwx   1 root root    0 Nov 22  2023 subsystem -> ../../../bus/platform

lrwxrwxrwx   1 root root    0 Jan  7 14:16 supplier:i2c:1-0042 -> ../../virtual/devlink/i2c:1-0042--platform:fdab0000.npu

lrwxrwxrwx   1 root root    0 Jan  7 14:16 supplier:platform:fd8d8000.power-management:power-controller -> ../../virtual/devlink/platform:fd8d8000.power-management:power-controller--platform:fdab0000.npu

lrwxrwxrwx   1 root root    0 Jan  7 14:16 supplier:platform:fdab9000.iommu -> ../../virtual/devlink/platform:fdab9000.iommu--platform:fdab0000.npu

lrwxrwxrwx   1 root root    0 Jan  7 14:16 supplier:platform:firmware:scmi -> ../../virtual/devlink/platform:firmware:scmi--platform:fdab0000.npu

lrwxrwxrwx   1 root root    0 Jan  7 14:16 supplier:regulator:regulator.37 -> ../../virtual/devlink/regulator:regulator.37--platform:fdab0000.npu

-rw-r--r--   1 root root 4096 Nov 22  2023 uevent

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 检查 rknnlite 是否已安装

root@talowe-rk3588:~#   python3 -c "from rknnlite.api import RKNNLite; print('RKNNLite OK')" 2>&1

Traceback (most recent call last):

File "<string>", line 1, in <module>

ModuleNotFoundError: No module named 'rknnlite'

root@talowe-rk3588:~#   # 1. 升级 pip

root@talowe-rk3588:~#   pip3 install --upgrade pip

Collecting pip

Downloading pip-25.0.1-py3-none-any.whl (1.8 MB)

|████████████████████████████████| 1.8 MB 688 kB/s

Installing collected packages: pip

Attempting uninstall: pip

Found existing installation: pip 20.0.2

Not uninstalling pip at /usr/lib/python3/dist-packages, outside environment /usr

Can't uninstall 'pip'. No files were found to uninstall.

Successfully installed pip-25.0.1

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 2. 安装依赖

root@talowe-rk3588:~#   pip3 install numpy opencv-python-headless

Collecting numpy

Downloading numpy-1.24.4-cp38-cp38-manylinux_2_17_aarch64.manylinux2014_aarch64.whl (14.0 MB)

|████████████████████████████████| 14.0 MB 627 kB/s

Collecting opencv-python-headless

Downloading opencv_python_headless-4.12.0.88-cp37-abi3-manylinux2014_aarch64.manylinux_2_17_aarch64.whl (33.1 MB)

|████████████████████████████████| 33.1 MB 320 kB/s

Installing collected packages: numpy, opencv-python-headless

Successfully installed numpy-1.24.4 opencv-python-headless-4.12.0.88

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 3. 安装 rknn-toolkit-lite2

root@talowe-rk3588:~#   pip3 install rknn-toolkit-lite2

Collecting rknn-toolkit-lite2

Downloading rknn_toolkit_lite2-2.3.2-cp38-cp38-manylinux_2_17_aarch64.manylinux2014_aarch64.whl (562 kB)

|████████████████████████████████| 562 kB 478 kB/s

Requirement already satisfied: numpy in /usr/local/lib/python3.8/dist-packages (from rknn-toolkit-lite2) (1.24.4)

Collecting ruamel.yaml

Downloading ruamel.yaml-0.18.16-py3-none-any.whl (119 kB)

|████████████████████████████████| 119 kB 10.3 MB/s

Collecting psutil

Downloading psutil-7.2.1-cp36-abi3-manylinux2014_aarch64.manylinux_2_17_aarch64.manylinux_2_28_aarch64.whl (156 kB)

|████████████████████████████████| 156 kB 3.8 MB/s

Collecting ruamel.yaml.clib>=0.2.7; platform_python_implementation == "CPython" and python_version < "3.14"

Downloading ruamel.yaml.clib-0.2.8.tar.gz (213 kB)

|████████████████████████████████| 213 kB 9.2 MB/s

Installing build dependencies ... done

Getting requirements to build wheel ... done

Preparing wheel metadata ... done

Building wheels for collected packages: ruamel.yaml.clib

Building wheel for ruamel.yaml.clib (PEP 517) ... done

Created wheel for ruamel.yaml.clib: filename=ruamel_yaml_clib-0.2.8-cp38-cp38-linux_aarch64.whl size=744421 sha256=3ad2438c644cbafc6a485cc6bf884c9392a98b9f813b1ce5d5bfaf1e15de6115

Stored in directory: /root/.cache/pip/wheels/01/da/7d/b9216e2302a1e02e860315cbd43ccc0d4233cdce0e122c85b4

Successfully built ruamel.yaml.clib

Installing collected packages: ruamel.yaml.clib, ruamel.yaml, psutil, rknn-toolkit-lite2

Successfully installed psutil-7.2.1 rknn-toolkit-lite2-2.3.2 ruamel.yaml-0.18.16 ruamel.yaml.clib

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 4. 验证安装

root@talowe-rk3588:~#   python3 -c "from rknnlite.api import RKNNLite; print('RKNNLite OK')"

RKNNLite OK

root@talowe-rk3588:~#  # 1. 查看两个网卡速度

root@talowe-rk3588:~#   ethtool eth0 2>/dev/null | grep -E "Speed|Link"

Speed: Unknown!

Link detected: no

root@talowe-rk3588:~#   ethtool eth1 2>/dev/null | grep -E "Speed|Link"

Link partner advertised link modes:  10baseT/Half 10baseT/Full

Link partner advertised pause frame use: Symmetric Receive-only

Link partner advertised auto-negotiation: Yes

Link partner advertised FEC modes: Not reported

Speed: 100Mb/s

Link detected: yes

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 2. 查看网卡状态

root@talowe-rk3588:~#   ip link show eth0

4: eth0: <NO-CARRIER,BROADCAST,MULTICAST,UP> mtu 1500 qdisc mq state DOWN mode DEFAULT group default qlen 1000

link/ether b6:4c:e9:3b:97:32 brd ff:ff:ff:ff:ff:ff

root@talowe-rk3588:~#   ip link show eth1

5: eth1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc mq state UP mode DEFAULT group default qlen 1000

link/ether ba:4c:e9:3b:97:32 brd ff:ff:ff:ff:ff:ff

root@talowe-rk3588:~#

root@talowe-rk3588:~#   # 3. 如果 eth0 没插网线，先跳过

root@talowe-rk3588:~#   # 检查是否安装 iperf3

root@talowe-rk3588:~#   which iperf3 || apt install -y iperf3

/usr/bin/iperf3

root@talowe-rk3588:~#   cd /root/rk-app && PYTHONPATH=/root/rk-app python3 apps/yolov8_rknn_infer.py \

>       --model artifacts/models/yolo11n_416.rknn \

>       --source assets/bus.jpg \

>       --save /tmp/result.jpg \

>       --imgsz 416 --conf 0.5

2026-01-07 14:47:00 - apps.utils.headless - INFO - Running in HEADLESS mode - cv2.imshow() will save to files

W rknn-toolkit-lite2 version: 2.3.2

2026-01-07 14:47:00 - __main__ - I - Loading RKNN: artifacts/models/yolo11n_416.rknn

2026-01-07 14:47:00 - __main__ - I - Initializing runtime, core_mask=0x7

I RKNN: [14:47:00.818] RKNN Runtime Information, librknnrt version: 2.3.2 (429f97ae6b@2025-04-09T09:09:27)

I RKNN: [14:47:00.818] RKNN Driver Information, version: 0.8.2

W RKNN: [14:47:00.818] Current driver version: 0.8.2, recommend to upgrade the driver to the new version: >= 0.8.8

I RKNN: [14:47:00.818] RKNN Model Information, version: 6, toolkit version: 2.3.2(compiler version: 2.3.2 (e045de294f@2025-04-07T19:48:25)), target: RKNPU v2, target platform: rk3588, framework name: ONNX, framework layout: NCHW, model inference type: static_shape

W RKNN: [14:47:00.829] query RKNN_QUERY_INPUT_DYNAMIC_RANGE error, rknn model is static shape type, please export rknn with dynamic_shapes

W Query dynamic range failed. Ret code: RKNN_ERR_MODEL_INVALID. (If it is a static shape RKNN model, please ignore the above warning message.)

2026-01-07 14:47:00 - __main__ - I - Inference time: 25.31 ms

2026-01-07 14:47:00 - __main__ - I - Detections: 25

2026-01-07 14:47:00 - __main__ - I - Saved: /tmp/result.jpg

root@talowe-rk3588:~/rk-app#